#include <iostream>

#include <cmath>

#include<iomanip>

using namespace std;

//parameters

//弹簧管

#define E 1.2e11	//弹性模量Pa

#define u 0.33		//泊松比

#define h 0.0008	//壁厚8mm

#define R 0.045		//中径45mm

#define r 240*3.1415926/180.0	//参加弹性变形部分中心角rad

#define p 1e6		//1MPa

#define c1 0.437	//扁平型截面，a/b=4

#define c2 0.121

#define a0 0.00976	//m

#define b0 0.00244	//m

#define pi 3.1415926

//曲柄滑块

#define fai0 (-9)*3.1415926/180.0		//rad

#define faiz 9*3.1415926/180.0			//rad

#define angle_fai0 (-9)

#define angle_faiz 9

#define lambda 4

#define epsilon 1

//齿轮传动机构

#define i12 15

#define angle_faig 18

//游丝

#define eat 0.9		//齿轮传动效率

#define K 2

#define sigmab 6e8		//强度极限

#define f 0.2

#define fv 0.2*1.57

#define Fr 0.172

#define Fa 0.172

#define d1 0.0015			//1.5mm

#define Fr_fan 0.0914

#define Fa_fan 0.0914

#define d2 0.003			//3mm

#define D1 0.031			//外径

#define D2 0.004			//内径

#define n 9					//圈数

#define u_race 6			//宽厚比

#define S 4					//许用安全系数

#define sigmab_std 1.5e8	//强度校验

double s, a, b1, fai1, delta, e;

double c, A, b;

double lambda_new;

void spring_tube();

void crank_block();

void gear();

void mistake();

void balance_spring();

double dream_num(double N);

double exact_num(double N);

double abs_percent(double sn, double sn1);

long double Mfz_cal(double fr, double fa);

int main() {

cout << fixed << setprecision(4);

cout << "----------弹簧管----------" << endl;

spring_tube();

cout << "----------曲柄滑块----------" << endl;

crank_block();

cout << "----------齿轮传动机构----------" << endl;

gear();

cout << "----------压力表原理误差----------" << endl;

mistake();

cout << "----------游丝----------" << endl;

balance_spring();

return 0;

}

//弹簧管

void spring_tube() {

double w, r1, sr, st;

w = (p * (1 - pow(u, 2)) * pow(R, 2) * c1 * (1 - pow(b0, 2) / pow(a0, 2))) /

(E * b0 * h * (c2 + pow(R, 2) * pow(h, 2) / pow(a0, 4)));

sr = w * R * (1 - cos(r));

st = w * R * (r - sin(r));

s = sqrt(pow(sr, 2) + pow(st, 2));

delta = atan(sr / st) * 180.0 / pi;

cout << "(γ-γ')/γ = " << w << endl;

cout << "切向位移St：" << st * 1000 << "mm" << endl;

cout << "径向位移Sr：" << sr * 1000 << "mm" << endl;

cout << "自由端位移S：" << s * 1000 << "mm" << endl;

cout << "切线夹角：" << delta << "°" << endl;

}

//曲柄滑块

void crank_block() {

double smax = s;

a = smax / (

(sin(faiz) - sin(fai0)) + (sqrt(pow(lambda, 2) - pow((cos(fai0) - epsilon), 2)) -

sqrt(pow(lambda, 2) - pow((cos(faiz) - epsilon), 2)))

);

b1 = lambda * a;

e = epsilon * a;

fai1 = (asin((e - a * cos(fai0)) / b1)) * 180.0 / pi;

cout << "曲柄长度a：" << a * 1000 << "mm" << endl;

cout << "连杆长度b1：" << b1 * 1000 << "mm" << endl;

cout << "偏距e：" << e * 1000 << "mm" << endl;

cout << "初始位置与滑块运动方向夹角φ'：" << fai1 << "°" << endl;

}

//齿轮传动机构

void gear() {

double c11, A1;

double angle_odb, angle_o1bo, oo1b, dob;

angle_odb = 180 - (angle_fai0 + (90 - fai1));		//epsilon = 1

c11 = sqrt(pow(a, 2) + pow(b1, 2) - 2 * a * b1 * cos(angle_odb * pi / 180.0));

angle_o1bo = 90 - delta - asin(e / c11) * 180.0 / pi;

A1 = sqrt(pow(R, 2) + pow(c11, 2) - 2 * R * c11 * cos(angle_o1bo * pi / 180.0));

cout << "过渡量：" << c11 * 1000 << "mm" << endl;

cout << "齿轮理论中心距：" << A1 * 1000 << "mm" << endl;

float m[4] = { 0.2, 0.25, 0.3, 0.4 };

float min_m = m[0];

double min_sol = 1;

int z1, z2, min_z1, min_z2;		//2->主动轮，1->从动轮

for (int i = 0; i < 4; i++) {

for (int j = 17; j < 30; j++) {		//避免根切

z1 = j;

z2 = z1 * i12;

A = m[i] * (z1 + z2) / 2;

if (abs(A - A1 * 1000) < min_sol) {

min_sol = abs(A - A1 * 1000);

min_m = m[i];

min_z1 = z1;

min_z2 = z2;

}

}

}

A = min_m * (min_z1 + min_z2) / 2 / 1000;			//m

cout << "最合适的模数：" << min_m << endl;

cout << "主动轮齿数：" << min_z2 << endl;

cout << "从动轮齿数：" << min_z1 << endl;

cout << "从动轮参数：" << endl;

cout << "分度圆直径d1：" << min_m * min_z1 << "\t\t" <<

"齿顶圆直径da1：" << int(min_m * (min_z1 + 2 * 1) + 1) << "\t\t" <<

"齿根圆直径df1：" << min_m * (min_z1 - 2 * 1 - 2 * 0.25) << endl;

cout << "主动轮参数：" << endl;

cout << "分度圆直径d2：" << min_m * min_z2 << "\t\t" <<

"齿顶圆直径da2：" << int(min_m * (min_z2 + 2 * 1) + 1) << "\t\t" <<

"齿根圆直径df2：" << min_m * (min_z2 - 2 * 1 - 2 * 0.25) << endl;

//连杆长度b修正

oo1b = acos((pow(R, 2) + pow(A1, 2) - pow(c11, 2)) / (2 * R * A1));

dob = acos((pow(a, 2) + pow(c11, 2) - pow(b1, 2)) / (2 * a * c11));

c = sqrt(pow(R, 2) + pow(A, 2) - 2 * R * A * cos(oo1b));

b = sqrt(pow(a, 2) + pow(c, 2) - 2 * a * c * cos(dob));

double lambda1 = b / a;

double angle_delta_fan, angle_gt;

angle_gt = (4 * pi / min_z2) * 180.0 / pi;

angle_delta_fan = angle_faig * 1.25 + 2 * angle_gt;

cout << "修正后：" << endl;

cout << "中间量c：" << c * 1000 << endl;

cout << "连杆长度b：" << b * 1000 << endl;

cout << "连杆相对长度：" << lambda1 << endl;

cout << "扇形截取大小δ扇：" << angle_delta_fan << endl;

lambda_new = lambda1;

}

//压力表原理误差,-9°~+9°

void mistake() {

cout << "序号\t" << "计算点\t" << "理想值\t" << "实际值\t" << "绝对值\t" << "相对误差" << endl;

for (int i = 0; i < 19; i++) {

cout << i + 1 << "\t" << angle_fai0 + i << "°\t" << dream_num(angle_fai0 + i) << "\t"

<< exact_num(angle_fai0 + i) << "\t" << abs(dream_num(angle_fai0 + i) - exact_num(angle_fai0 + i))

<< "\t" << abs_percent(dream_num(angle_fai0 + i), exact_num(angle_fai0 + i))

<< "%" << endl;

}

}

double dream_num(double N) {

double sn = ((N - angle_fai0) * s / (angle_faiz - angle_fai0)) * 1000;

return sn;

}

double exact_num(double N) {

double sn1 = (a * (sin(N * pi / 180.0) - sin(angle_fai0 * pi / 180.0) +

sqrt(pow(lambda_new, 2) - pow((cos(fai0) - epsilon), 2)) -

sqrt(pow(lambda_new, 2) - pow((cos(N * pi / 180.0) - epsilon), 2)))) *

1000;

return sn1;

}

double abs_percent(double sn, double sn1) {

return abs(sn - sn1) * 100 / sn;

}

//游丝

void balance_spring() {

double Mfz1, Mfz2, Mfz;

Mfz1 = Mfz_cal(Fr, Fa);

Mfz2 = Mfz_cal(Fr_fan, Fa_fan);

cout << "小齿轮：" << Mfz1 * 1e6 << "μN·m" << endl;

cout << "扇形齿轮：" << Mfz2 * 1e6 << "μN·m" << endl;

Mfz = Mfz1 + Mfz2 / (i12 * eat);

cout << "Mfz：" << Mfz * 1e6 << "μN·m" << endl;

double Mmin = K * Mfz / (1 - 0.1 * K);

cout << "最小力矩Mmin：" << Mmin * 1e6 << "μN·m" << endl;

double L = pi * n * (D1 + D2) / 2;

double height = sqrt(sqrt(12 * L * Mmin / u_race / E / (pi / 2)));

//检验长度、宽度、厚度是否满足强度条件

cout << "长度：" << L * 1000 << "mm" << endl;

//圆整至0.01,向上圆整

float h_last = ceil(height * 1e5) / 100;

cout << "厚度：" << fixed << setprecision(2) << h_last << "mm" << endl;

float b_last = h_last * u_race;

cout << "宽度：" << fixed << setprecision(2) << b_last << "mm" << endl;

double sigma_max, Mmax;

Mmax = 4 * Mmin;		//2pi/(pi/2) = 4, Pa

sigma_max = 6 * Mmax / (b_last / 1000 * pow(h_last / 1000, 2));

cout << "强度σb：" << sigma_max / 1e6 << "MPa";

//如果满足，进行最终确定L, n, a

if (sigma_max <= sigmab_std) {

cout << " < 150MPa\t符合要求" << endl;

//长度

double l_last = E * b_last / 1000 * pow(h_last / 1000, 3) * pi / 2

/ (12 * Mmin);

//圈数

double circle_cnt = 2 * l_last / (pi * (D1 + D2));

//圈间距

double a_dis = (D1 - D2) / 2 / circle_cnt;

//加工同时盘绕个数

int k_cnt = a_dis / (h_last / 1000) - 0.5;		//向下取整

cout << "长度：" << l_last * 1000 << "mm" << endl;

cout << "圈数：" << circle_cnt << "圈" << endl;

cout << "圈间距：" << a_dis * 1000 << "mm" << endl;

cout << "加工同时盘绕个数：" << k_cnt << "个" << endl;

}

}

long double Mfz_cal(double fr, double fa) {

long double M_vertical, M_level;

M_vertical = 0.5 * fv * fr * d1;

M_level = f * fa / 3 * (pow(d1, 3) - pow(d2, 3)) /

(pow(d1, 2) - pow(d2, 2));

return max(M_vertical, M_level);

}